1. RTL Design (alu.v)

module alu (
    input  [7:0] A,
    input  [7:0] B,
    input  [2:0] ALU_Sel,
    output reg [7:0] Result,
    output Zero
);

always @(*) begin
    case(ALU_Sel)

        3'b000: Result = A + B;        // Addition

        3'b001: Result = A - B;        // Subtraction

        3'b010: Result = A & B;        // AND

        3'b011: Result = A | B;        // OR

        3'b100: Result = A ^ B;        // XOR

        3'b101: Result = ~A;           // NOT

        3'b110: Result = A << 1;       // Left Shift

        3'b111: Result = A >> 1;       // Right Shift

        default: Result = 8'd0;

    endcase
end

assign Zero = (Result == 8'd0);

endmodule

2. Testbench (alu_tb.v)

`timescale 1ns/1ps

module alu_tb;

reg [7:0] A;

reg [7:0] B;

reg [2:0] ALU_Sel;

wire [7:0] Result;

wire Zero;

integer i;

alu DUT (

.A(A),

.B(B),

.ALU_Sel(ALU_Sel),

.Result(Result),

.Zero(Zero)

);

initial begin

$dumpfile("alu.vcd");

$dumpvars(0, alu_tb);

//-------------------------------

// Directed Test Cases

//-------------------------------

A = 10; B = 5; ALU_Sel = 3'b000;

#10;

A = 10; B = 5; ALU_Sel = 3'b001;

#10;

A = 8'hAA; B = 8'h55; ALU_Sel = 3'b010;

#10;

A = 8'hAA; B = 8'h55; ALU_Sel = 3'b011;

#10;

A = 8'hAA; B = 8'h55; ALU_Sel = 3'b100;

#10;

A = 8'h0F; B = 0; ALU_Sel = 3'b101;

#10;

A = 8'h08; B = 0; ALU_Sel = 3'b110;

#10;

A = 8'h80; B = 0; ALU_Sel = 3'b111;

#10;

//--------------------------------

// Random Test Cases

//--------------------------------

for(i=0;i<100;i=i+1)

begin

A = $random;

B = $random;

ALU_Sel = $random % 8;

#10;

end

$finish;

end

endmodule

Coverage achieved:

- Directed operation coverage: 100% (all 8 ALU operations exercised)

- Random testing: 100 randomized input combinations across all operations

- Corner cases covered: Zero result, alternating bit patterns (0xAA, 0x55), shift operations, and arithmetic operations

3. Simulation

Using ModelSim

Compile :-

vlog alu.v
vlog alu_tb.v

Run :-

vsim alu_tb
run -all

View waveform :-

add wave *

Using Icarus Verilog + GTKWave

Compile :-

iverilog -o alu alu.v alu_tb.v

Run :-

vvp alu

Open waveform :-

gtkwave alu.vcd

Coverage achieved:

- Directed operation coverage: 100% (all 8 ALU operations exercised)

- Random testing: 100 randomized input combinations across all operations

- Corner cases covered: Zero result, alternating bit patterns (0xAA, 0x55), shift operations, and arithmetic operations

waveform