TIMING REPORT :-

| Parameter | Value | Status |
| --- | --- | --- |
| Tool | OpenSTA 2.5.0 | - |
| Design Name | alu | - |
| Technology | Generic 45 nm Std Cell | - |
| Process Corner | TT (Typical) | - |
| Voltage | 1.0 V | - |
| Temperature | 25°C | - |
| Clock Name | clk | - |
| Clock Period | 10.000 ns | ✓ |
| Clock Frequency | 100 MHz | ✓ |
| Clock Uncertainty | 0.100 ns | ✓ |
| Clock Latency | 0.200 ns | ✓ |
| Data Arrival Time | 6.845 ns | ✓ |
| Required Arrival Time | 9.900 ns | ✓ |
| Setup Slack | +3.055 ns | PASS |
| Hold Slack | +0.420 ns | PASS |
| Worst Negative Slack (WNS) | 0.000 ns | PASS |
| Total Negative Slack (TNS) | 0.000 ns | PASS |
| Setup Violations | 0 | PASS |
| Hold Violations | 0 | PASS |
| Overall Timing Status | PASSED | ✓ |

VIOLATION TIMING :-

| Parameter | Value | Status |
| --- | --- | --- |
| Clock Period | 5.000 ns | - |
| Data Arrival Time | 5.380 ns | - |
| Required Arrival Time | 5.000 ns | - |
| Setup Slack | -0.380 ns | FAIL |
| Hold Slack | +0.120 ns | PASS |
| Worst Negative Slack (WNS) | -0.380 ns | FAIL |
| Total Negative Slack (TNS) | -1.420 ns | FAIL |
| Violating Paths | 4 | FAIL |
| Overall Timing Status | FAILED | ✗ |

How to Fix Timing Violations

If setup violations occur (negative setup slack):

- Pipeline the datapath by adding registers to shorten combinational paths.

- Reduce logic depth by simplifying arithmetic or logic expressions.

- Increase the clock period (lower the operating frequency) if performance requirements allow.

- Re-synthesize with higher optimization effort (e.g., abc or timing-driven optimization).

- Use faster standard cells from the timing library (where available).

If hold violations occur (negative hold slack):

- Insert delay buffers on short data paths.

- Increase hold margins using synthesis or P&R optimization.

- Adjust clock skew through clock-tree synthesis.